RESPONSE FORM - Public Health & Housing

| Planning West Suffolk Council West Suffolk House Western Way Bury St Edmunds Suffolk |  |  |  | Case Officer: |  |  | Kerri Cooper, DC/20/0094/RM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Application No: |  |  | DC/20/0094/RM |  |  |  |
|  |  |  |  | Consultation Period Expires: |  |  | 20/02/2020 |  |  |  |
| Regulatory Officer (Name): |  |  |  | Emma Forsberg |  |  |  |  |  |  |
| Regulatory Reference Number: |  |  |  | WK/202003150 |  |  |  |  |  |  |
| CONSULTATION ON APPLICATION RECEIVED BY LOCAL PLANNING AUTHORITY |  |  |  |  |  |  |  |  |  |  |
| PROPOSAL: |  | Reserved Matters Application - Submission of details under Outline Planning Permission DC/15/2424/OUT - Matters Reserved by Condition 2 appearance, landscaping, layout and scale for the development of Units 1 and 2 for Class B2 and B8 use. Application |  |  |  |  |  |  |  |  |
| LOCATION: |  | Land Adj Haverhill Business Park, Bumpstead Road, Haverhill, Suffolk |  |  |  |  |  |  |  |  |
| Sections(s) responding (please tick): |  |  |  |  |  |  |  |  |  |  |
| Contaminated Land |  |  | Local Air Quality |  |  | Private Water Supplies |  |  |  |  |
| Sustainable Development |  |  | Environmental Permitting Issues |  |  | Commercial Environmental Health |  |  |  |  |
| Licensing |  |  | Public Health & Housing |  |  |  |  |  |  |  |
| Do the Public Health & Housing object to this Application? |  |  |  |  |  |  |  | Yes | No |  |
| Comments to explain why object/support the application: The noise report dated 8 January 2020 Project No. 1919017 produced by Sharps Redmore to support this application, in short, demonstrates that the impact from noise is reduced compared to that previously approved layout under DC/19/1010/RM. The noise report does refer to their previous report dated 19 July 2019, submitted to support the approved reserved matters application DC/19/1010/RM and is a helpful reference to understand the noise data and conclusions made in the 2020 report. The two areas of concern are internal and plant noise and external noise activities. It is the combination of these that will define the overall impact on the closest noise sensitive receptors. Internal activities and plant noise The noise report does not provide any detail on the proposed noise impact of internal or plant noise. The information is not available as the end user and exact plant and activity is unknown. The 2019 noise report, the assessment in section 5, assumes that the units will be constructed using a composite metal panel system providing a typical sound reduction performance of 28 Rw. The impact of noise from within the units will depend on the specific use, however, based on those assumptions accepted noise from the internal activities will be below the existing representative day and night-time levels at the nearest noise sensitive premises. The 2020 report does go into detail on the existing condition to control noise (condition 3) from internal activities and plant noise and argues at length that it is an unreasonable condition. This is based on NPPG guidance that specifies what criteria would fall under significant impact (+ 10dB above background) and WHO guidelines and BS2833 (itemised in Table 2 of the report) maintaining internal and external noise levels to avoid impact to residents health e.g. night-time limit within a bedroom is 30dB LAeqT with a max sound of 45dB AMAX. External daytime limit e.g. in the garden 50dB LAeqT. Ultimately there is no new information provided regarding internal and plant noise based on the new layout. External activities Car parking The 2020 report concludes that the number of car parking spaces has been reduced and location of some of the car parking places are further away from the sensitive noise receptors. The change in layout should have a reduced impact from car parking. Unloading The 2020 report looks at the units operated 7 days a week 24 hours a day but assumes only 50% of bays would be in use at night-time. This was the same assumption as the previous noise assessment that supported DC/19/1010/RM approved at that time. This was queried at the time and the worst case with 100% of bays open and in use increased the noise level by 2dB. Following a change in layout the predicted noise impact is calculated to be lower than already permitted. The report goes on to conclude that a condition should be in place to require that before any phase or unit of the development is occupied a Management Plan for the phase or unit, including hours of operation, hours of deliveries, and full details of loading/unloading arrangements along with any noise mitigation necessary are submitted to the Council. I understand this has been conditioned under DC/19/1010/RM – condition 4. The reality of noise means those high-level short duration noise impacts are averaged out, which is why we also look at peak noise levels likely. Both the average and peak noise predicted from external activity in isolation are shown to be within the level of WHO guidelines and daytime background levels. Predicted noise levels from service activity are shown to be at or slightly above night-time background levels. Although it is made clear in the report that the existing noise condition 3 of DC/19/1010/RM is considered unreasonable, it is limited to internal and plant noise only which is constant and arguably more controllable. The impact to nearby residents is accumulative noise generated from internal activity, plant and external activity (from unloading and loading, vehicle movements etc). Condition 3 (DC/19/1010/RM) is an existing condition. The combination of internal, plant and external noise rated levels is the true impact to nearby residents which cannot be demonstrated at this time. Whilst the prediction for external noise in isolation does suggest noise from the units is at background or close to it at noise sensitive receptors (levels suggested as reasonable to be conditioned (40dB daytime, 35dB night-time background), in the noise reports), I do not suggest amending or weakening this condition. By ensuring plant and internal activity is -10dB below background (in isolation from external activity) the applicant and planning authority can be confident that the combined noise from all activities (based on those assumptions made) will have a low impact on nearby residents. |  |  |  |  |  |  |  |  |  |  |
| Suggested conditions or amendments: |  |  |  |  |  |  |  |  |  |  |
| Observations on non planning issues: |  |  |  |  |  |  |  |  |  |  |
| Date: 31 July 2026 |  |  |  |  |  |  |  |  |  |  |