### FLOOD RISK ASSESSMENT

OF

FLUVIAL FLOODING RISKS

AT

THE IRON WORKS SITE,

HAMLET ROAD, SUFFOLK.

#### Contents: -

- Introduction.

- Existing site details and proposed development.

- Flooding History.

- Flood risks.

- Conclusions.

Appendices: -

- Location Plan.

- Environment Agency Indicative Flood Map.

- Extract from the Environment Agency’s North Essex Catchment Flood Management Plan – Scoping Report.  Part 3 Catchment Flood Risk.

- Rainfall Runoff Calculation Output.

- Rectangular channel calculations u/s weir.

- Rectangular channel calculations d/s weir.

- Site survey.

- Proposal Drawing.

1.0	INTRODUCTION

1.1	This flood risk assessment has been prepared in accordance with the recommendations contained in Planning Policy Guidance Note 25 “Development and Flood Risk” (PPG 25) in accordance with St Edmundsbury Borough Council’s Policy requiring Flood Risk Assessments to accompany planning submissions.

2.0	EXISTING SITE DETAILS AND PROPOSED DEVELOPMENT

  - Ehringshausen Way and Hamlet Road bound the site to the northeast and south respectively.  The canalised Main River Stour nominally bisects the site. The site centroid is at National Grid Reference TL 67800 44925.  Refer to Appendix A for site location and Appendix G for site survey.

  - The proposed development is to completely demolish the existing Engineering Works and to redevelop the whole site for residential dwellings, parking and associated circulation areas.

  - The proposed development will not increase the impermeable area of the site and surface water discharges will not modify the land drainage status quo.

  - Irrespective of other consents, any works in, over, or under and within 9m of the channel edge of the Main River Stour will require the prior approval of the Environment Agency under Section 109 of the Water Resources Act 1991.  Such a consent cannot be reasonably withheld and the Environment Agency has indicated that the proposed works, provided the river bed and cross section area are maintained unaltered, will most probably receive the necessary prior consent.

3.0	FLOODING HISTORY

3.1	The proposal site is shown by the Environment Agency to be partly within an area

of indicative flood risk.

  - Maximum flood levels at the site have not been formally recorded; however the 1968 flooding of the site is reported by St Edmundsbury Borough Council “ The History of Haverhill in the 20th Century” as follows: -

| 1968 | Ten years after the great Haverhill floods of 1958, despite new drainage levies, floods occurred again in mid September 1968. Some 2.7 inches of rain fell in 24 hours, and much of Southern Britain was also under water. The Haverhill Echo of Thursday 19th September said "they were if anything, greater in intensity than those of ten years ago". It rained all the weekend and on Monday 16th September the town was flooded "to the knee" as the Stour Brook overflowed. Withersfield Road, Queen Street, The Pightle and Hamlet Road were all flooded. "The Relief Road (Ehringshausen Way) resembled a canal". The Council Offices in Lower Down Slade were flooded and records in the cellar were destroyed. By mid morning the water had subsided but there was a thick layer of slushy mud left behind clinging to floors and walls. |
| --- | --- |

  - Since the 1968 flooding Haverhill has received flood protection in the form of the

Meldham Washland. Refer to Appendix C.  C the North Essex Flood Management

Plan – Scoping Report.  The Scoping Report (page 56) indicates that the October 2001 flood event was contained by the Meldham Washland with no flooding of residential properties in Haverhill.  The Scoping Report 3.5.4 Flood Storage states that ‘the Meldham Washland is an important storage area because it is of a size and position to influence flood conditions on a significant scale.

4.0	FLOOD RISKS

4.1	The River Stour catchment has been assessed for peak flow at the site and a flow, without the benefit of flow reductions by way of the Meldham Washland, is predicted at 13.4m3/s for the 1:100 year flood flow with a design rainfall of 59mm in 25 hours.  Refer to Appendix D for rainfall runoff calculation sheets. The historic flooding event mentioned at point 3.2 gives the associated rainfall as 2.7” in 24 hours, this is nominally 68mm in 24 hours for a major event and fits well with the 1:100 predicted rainfall of 59mm in 25 hours.  The 68mm rainfall being closer to a 1:200 year return period flood event.

  - The Scoping Report Table 3.4 (page 60) summarises return period peak flows and gives an inflow of 14.02m3/s for the 1:100 year return period.  Anomalously, the Scoping report summarises the outflows as greater than the inflows.  The Environment Agency has kindly supplied the origin of the Scoping Report and this is attached as Appendix I, for the purposes of this assessment, the outflow from the downstream boundary is taken at 14.42m3/s.  From the peak flow calculated at point 3.4, it is considered sufficiently conservative to use the 14.42m3/s flow to assess the River Stour conveyance at the site under review.

  - Within the site, the River Stour is canalised to a rectangular concrete channel 2.9m wide and minimum 2.0m deep.  The channel in this configuration represents the smallest available local cross-section to available to river flows.  The average riverbed gradient to the relevant river reach is 0.003m/m with minimum bank levels    supporting this gradient.  The River Stour passes over an amenity weir within the site area and rapidly expands in cross-sectional area and the bed level drops by nominally 0.5m with the bed gradient reducing to .0015m/s for the immediate downstream section.  The conveyance capacities of the River Stour have been assessed both upstream and downstream of the amenity weir and the respective capacities are given at 21.6m3/s and 37.8m3/s respectively.

  - From a ‘walk through’ survey along the relevant downstream section of the River Stour it is apparent that the cross-sectional area of the river remains greater than that within the site and upstream of the amenity weir.

  - The Environment Agencies Flood Defence Staff advise that the River Stour through Haverhill is susceptible to blockages, primarily caused by fly tipping of objects into the River, and to counter this risk the Agency operates a monthly inspection with appropriate remedial works on a permanent basis.

  - The River Stour, at the site under review, has sufficient conveyance capacity to contain peak 1:100 year return period flows within banks.  The risks of blockages are practically resolved by the Environment Agency’s high standard of maintenance given to the River Stour.  The site does not form part of the River Stour operational floodplains and the redevelopment will not modify the land drainage status quo.

  - Residual risks of flooding will remain, these from local drainage failures, and to ensure that dwellings remain free from such risks, a minimum ground floor level 300mm above the local crown of Ehringshausen Way highway is proposed at 49.50m to allow local sufficient scope for weir flows over the highway without detriment to the dwellings.  The proposed minimum finished floor level of 49.50m may be readily accommodated within the ‘normal’ construction of the site.

5.0	 SURFACE AND FOUL WATER DRAINAGE OF THE SITE.

  - The development will use established foul water Utility sewers serving the site.

  - The development will use established surface water connections serving the site to the River Stour.

6.0	CONCLUSIONS

  - Guidance for the requirements in undertaking a Flood Risk Assessment is contained in Planning Policy Guidance Note 25, Appendix F. Paragraph F.3 indicates the general requirements that should be considered for all Flood Risk Assessments irrespective of the size of the site and each requirement is commented upon as follows:  -

F3. 1 	A location plan is included with this assessment together with plans that show the site in more detail with site levels.

F3.2	The attached plans show existing ground levels that will be generally maintained.  Floor levels will be at minimum level of 49.50m

F3.3	The flood alleviation scheme serving Haverhill, The Meldham Washland, is mentioned in the assessment.

F3.4	The source of potential flooding is fluvial, which is commented upon.

F3.5	The Environment Agency’s indicative flood plain map is attached to this Assessment.

F3 .6	The conveyance capacities of the River Stour along with local features are considered and commented upon.

F3.7     Flooding probabilities and trends are commented upon.

F3.8 	Site levels are provided and floor levels are commented upon.

F3.9 	Not Applicable.

F3. 10 	Not Applicable.

F3. 11 	Not Applicable.

F3. 12	Not Applicable

F3. 13 	Not Applicable.

F3.14 	Allowances for potential climate changes and associated increased flows are

considered to be completely within the capacities of the Meldham Washland

as the flow reduction benefits have not be taken for this assessment.

F3. 15	 Not Applicable.

  - The River Stour has sufficient capacity to convey the 1:100 year peak flood event.

  - The high level of maintenance to the River Stour ensures that effective blockages are unlikely to occur.

  - The proposed redevelopment will present the same, if not less, impermeable surfaces as the existing site.

  - As the River Stour is properly maintained and able to convey the predicted 1:100 return period + 20% flow, with the considerable benefits given by the Meldham Washland, it is considered that the proposed development with minimum finished floor levels at 49.50m will not to be at flood risk as described by the parameters of Planning Policy Guidance 25: Development and flood risk.

…………………………………A D Rich. Associate.

For G H Bullard and Associates

Ref:  124/2004FRA