AP Landscapes Tree & Grounds Maintenance

1 Shadowbush

Stansfield Rd

Poslingford

CO10 8RE

07852239848

info@apgardens-trees.com

www.apgardens-trees.com

West Suffolk Council

Atten: Alice Whitbread

Ref: Planning invalidity notification - DC/25/1691/TPO, Planning Portal No: PP-14274939

Tree location: unit 15 Homefield road Haverhill CB9 8QP

Grouping of 5 Trees –

Field Maple – Acer Campestre

Height approx. 6m canopy spread of approx. 4.5m located outside of palisade fencing, adjacent to public foot path and main road.

X1 of the group has declined over the year, has necrosis & decay throughout the canopy and main structural wood.

Physiological condition: poor

Structural condition: poor

Risk Level: High

Recommendations: Fell to ground level

No fruiting bodies, fungus or bracket fungus observed.

Images taken 9th October 2025

Remaining trees will benefit from minor crown lifting giving clearance to public footpath security fencing.